Instructions for running simulation files

Example 3.1

- Run the MATLAB file Leaf_Initialization.m

- Then, run the Simulink file ‘Leaf_UDDS_Cycle’ to observe the Torque and Power output (scopes are within the ‘PMSM’ block).  The spreadsheet file ‘LA4_cycle.xlsx’ must be in the same folder as the simulation files.

- Finally, run the ‘Battery_Model_Leaf’ Simulink file to observe the Battery energy and SOC.

Example 3.2

- Run the MATLAB file Tesla_Initialization.m

- Then, run the Simulink file ‘Tesla_Hwy_Cycle’ to observe the Torque and Power output (scopes are within the ‘PMSM’ block). The spreadsheet file ‘Highway_cycle.xlsx’ must be in the same folder as the simulation files.

- Finally, run the ‘Battery_Model_Tesla’ Simulink file to observe the Battery energy and SOC.

Example 8.1

- Run the MATLAB file Tesla_Initialization.m

- Then, the Simulink file ‘PMSM_dq_control_MTPA’ to observe the Torque and motor currents (scopes are within the ‘PMSM’ block). The spreadsheet file ‘Highway_cycle.xlsx’ must be in the same folder as the simulation files.

Example 9.1

Note: To run this simulation, PSIM (available from Powersim: https://powersimtech.com/) must be installed.

- If the thermal model of the target SiC device (Wolfspeed CAB450M12XM3) is not already included in the PSIM Device Database, use the ‘cab450.dev’ file to add. Please refer to PSIM User Manual for details.

- Open the PSIM file ‘VSI_Thermal.psimsch’. Check Simulation Control parameters for appropriate timestep and other settings.

- Run the simulation

- After completing of simulation, open relevant waveforms from SimView.

Example 10.2

Note: To run this simulation, PLECS Blockset package (available from Plexim: https://www.plexim.com/download/blockset) must be installed.

- Run the MATLAB files: i) DeviceParam.m; and ii) Leaf_Initialization.m;.

- The spreadsheet file ‘Highway_cycle.xlsx’ must be in the same folder as the simulation files.

- Run the Simulink file ‘MotorDriveLoss.slx’ to observe the motor drive outputs (Torque, Speed, Id/Iq and Vd/Vq currents) and the Inverter outputs (FET rms currents, device losses (conduction, switching and total).

- The total losses are saved in two ‘.mat’ files for comparison: i) ‘PLoss_inv_Si.mat’; and ii) ‘PLoss_inv_Sic.mat’

Example 12.1

Run the MATLAB file Problem12_1.m for the calculations and generating the plots.